Circuit Test NTD file creation:

1.  Extract Netlist

2.  Output IPC-D-356

3.  Write PDF from ALL conductive layers using NTD.PDF.  Layer selection

example would be F1-F4 to output the traces for a four layer board.

You should exclude negative layers from the output list, since traces

in a negative layer are net zero.

Notes:

1.  NTD.PDF will output all traces with a 5 mil width.

2.  Converting poured planes to polygons will reduce the size of the NTD

file.