TMS320C2000 Code Composer Studio™ Flash Plug-in v1.10.2 Release Notes

These TMS320C2000 Code Composer Studio Flash Plug-in v1.10.2 release notes contain valuable information which may not be included in other product documentation. This information will assist you in the use of Code Composer Studio.

To keep up-to-date on specific items in this list or for newly published errata, visit www.ti.com/edsperrata. Use the unique identifier (SDSsq number) to find what you are looking for. If you wish to track the resolution or changes to the status of any item found on this page, simply add it to your profile using the myWorkspace feature on TIs website and you will automatically receive email notifications when changes occur.

Windows, Windows 98, Windows ME, Windows XP, Visual SourceSafe, Visual Basic, and Visual C++ are all trademarks and/or registered trademarks of Microsoft Corporation.

InstallShield and DemoShield are registered trademarks and service marks of InstallShield Software Corporation.

C2000, TMS320, TMS470, Code Composer Studio, DSP/BIOS, eXpressDSP, XDS560, and XDS510 are trademarks of Texas Instruments.

## Information about this update

This update installs both the F24xx Flash Plug-In and the F28xx Flash Plug-In. There are no changes to the F24xx Flash Plug-In in this update. The update for the F28xx Flash Plug-In supplies the user with a new Flash28xx.ocx plug in file.

The new Flash28xx.ocx plug in has no user visible changes, but does contain two bug fixes.

## Fixed Problems

| Identifier | Headline | Component | Target |
| --- | --- | --- | --- |
| SDSsq37292 | Flash plugin does not handle COPY section properly (-cr link option) | C2000 On-chip Flash Programmer | C2800 |
| The C28x CCS Flash Programmer Plugin appears to handle the .cinit section (COPY section) generated using the -cr linker option incorrectly. |  |  |  |
| SDSsq37792 | Having more than 100 sections causes Flash not to program. | C2000 On-chip Flash Programmer | C2800 |
| Having more than 100 sections causes Flash not to program. |  |  |  |

## Known Problems

| Identifier | Headline | Component | Target |
| --- | --- | --- | --- |
| SDSsq33887 | CCS On-Chip Flash plug-in for LF240xA does not work on Windows 98 | C2000 On-chip Flash Programmer | C2400 |
| The F24xx Flash Plug-In does not work on a Windows 98 platform. |  |  |  |
| SDSsq34047 | Placement of initialized data sections (like .const) | C2000 On-chip Flash Programmer | C2400 |
| Initialized data sections (like .const) need to be placed in flash program space in order to be initialized correctly. The F24xx flash programmer is not responsible for initializing data space. Refer to section 6.1.3 Allocating .const to Program Memory on page 6-5 of the TMS320C2x/C2xx/C5x Optimizing C Compiler User's Guide (SPRU024E) for details. This manual is located in ..\docs\pdf directory under your Code Composer installation directory. F24xx Flash Plug-in. |  |  |  |
| SDSsq34229 | F24xx flash plugin crashes when overinstalling the newer flash plugin | C2000 On-chip Flash Programmer | All |
| Installing the TMS320C2000 Code Composer Studio FLASH Plug-In v1.10 Flash plug-in patch over an existing flash plug-in installation results in initial failures of the F24x Flash plug-in v1.10. Workaround: If you have previously installed the TMS320C2000 Code Composer Studio FLASH Plug-In v1.00 patch, you must uninstall it prior to installing the v1.10 update. |  |  |  |
| SDSsq34046 | Breakpoints in user code will prevent flash programming | C2000 On-chip Flash Programmer | C2800 |
| If you have breakpoints set in your code, you must disable all breakpoints before clicking the Execute Flash button. Failure to do so will result in a warning that your source line breakpoints could not be removed. The Flash API code will be loaded, but will not be executed because of this error. You should run the target so that the Flash API code executes and performs the flash operation you have chosen. Be aware that If you have set a breakpoint in your program at an address that is in the same location that the Flash API is now loaded, the target will halt there while executing the Flash API code. You must run the target again so the Flash API code can complete the flash operation. Workaround: The workaround for this problem is to turn on the CCS option to clear all breakpoints when loading new programs. From the Option:Customize menu dialog, select the "Program Load Options" tab. Check the option "Clear All Breakpoints When Loading New Programs". |  |  |  |
| SDSsq34222 | "Execution" displays confusing pop-up. | C2000 On-chip Flash Programmer | C2800 |
| After hitting the "Execute Operation" button, a message appears asking the user to locate the file "FlashAPIInterface.c". Workaround: The workaround for this problem is for the user to turn off the CCS option to look for source code when debugging as follows. - Select the "Option:Customize" menu item. - Under the Editor Properties tab, deselect the checkbox for the "Enable file browse while debugging" option. |  |  |  |
| SDSsq34227 | Error msg appears twice when exiting CCS. | C2000 On-chip Flash Programmer | C2800 |
| Upon exiting Code Composer Studio, the user may receive two warnings about not being able to remove breakpoints. Workaround: The workaround for this problem is to not turn on the option to remove all breakpoints when loading a new program. To do so: - Select the Options:Customize menu item. - Click on the Program Load Options tab. - Check the "Clear All Breakpoints When Loading New Programs" option. - Click "OK" to dismiss the Customize dialog box. |  |  |  |

## Installation Details

F24xx Flash Plug-In

The TMS320C2000 Code Composer Studio Flash Plug-in v1.10.2 installs the same F24xx Flash Plug-In files as the TMS320C2000 Code Composer Studio Flash Plug-in v1.10 install.

F28xx Flash Plug-In

The TMS320C2000 Code Composer Studio Flash Plug-in v1.10 install placed the following files on the user’s system:

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2810\FlashAPIInterface2810V1_0Beta.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2811\FlashAPIInterface2811V1_0Beta.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2812\FlashAPIInterface2812V1_0Beta.out

<InstallDir>:\ti\plugins\Flash28xx\Flash28xx.ocx, File Version 1.0.0.2.

The TMS320C2000 Code Composer Studio Flash Plug-in v1.10.1 install placed the following files on the user’s system:

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2810\FlashAPIInterface2810V1_0.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2810\FlashAPIInterface2810V1_0Beta.out

This file is identical to the one installed by the previous release.

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2811\FlashAPIInterface2811V1_0.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2811\FlashAPIInterface2811V1_0Beta.out

This file is identical to the one installed by the previous release.

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2812\ FlashAPIInterface2812V1_0.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2812\ FlashAPIInterface2812V1_0Beta.out

This file is identical to the one installed by the previous release.

<InstallDir>:\ti\plugins\Flash28xx\Flash28xx.ocx, File Version 1.0.0.4

The install replaces Flash28xx.ocx, File Version 1.0.0.2.

The TMS320C2000 Code Composer Studio Flash Plug-in v1.10.2 install places the following files on the user’s system:

The following files are identical to the ones installed by the previous release:

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2810\FlashAPIInterface2810V1_0.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2810\FlashAPIInterface2810V1_0Beta.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2811\FlashAPIInterface2811V1_0.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2811\FlashAPIInterface2811V1_0Beta.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2812\ FlashAPIInterface2812V1_0.out

<InstallDir>:\ti\plugins\Flash28xx\Algorithms\2812\ FlashAPIInterface2812V1_0Beta.out

The install replaces Flash28xx.ocx, File Version 1.0.0.4.

<InstallDir>:\ti\plugins\Flash28xx\Flash28xx.ocx, File Version 1.0.0.5