Goldvision Mobile Surveillance

Version V1.0.1

This software supports mobile surveillance by PDA or smart phones.

To use mobile surveillance, your mobile device needs to be able to connect to a DVR through the network and support JAVA CLDC 1.0 and MIDP 2.0.

1. Functions

- Device Management

. Add or delete a DVR

. Edit the properties of a DVR such as the name, the IP address, the IP port, the user name and password.

. Save the DVR information permanently on the mobile device, so you don’t need to input it again.

- Live view the video

. Login a DVR

. Live view the video stream sent from the DVR

- Live record the video

.   Login a DVR

.  Live record the video stream to the mobile device

- Remote control cameras

. You can remote control cameras as you view the video. The controls are panning, zooming, focusing and iris.

2. Specification

Compression format:  MPEG 4

Display Resolution: 176x144(QCIF)

320x240(SCIF)

Display Frame Rate:  5 fps

Record Frame Rate:   10 fps

3. Installation

- Copy CellMonitor.jad and CellMonitor.jar to the mobile device.

- Open the CellMonitor.jad to install the software.

- Select the storage (Memory or extended Memory card) to install. If you select “Memory card”, please ensure that the card is not encrypted.

- When the mobile device prompts “installation completed”, you installed the software successfully.

4. Main Menu

Main Menu

- Browse devices

Select “Browse devices” item and click on “OK”, the software displays the device list. You can add, delete and edit the device through the device list.

- language

Select “language” item and click on “OK”, the software shows the language setup menu. You can change the language through the setup menu.

- about

Select “about” item and click on “OK”, the software shows the information of this software.

- help

Select “help” item and click on “OK”, the software shows the help document of this software.

5. Device List

When you select “Browse devices” item and click on “OK”, the device list shows up.

Device List

You can add, delete a device (DVR) or edit the properties of a device by using the device list.

(1) Add a device (DVR)

Step 1:

When you select “Options” under the device list (clicking on the button under “Options” item), a menu is displayed. The “Add Device” item is used to add a device.

Device List Menu

Step 2:

Selecting the “Add Device” item and clicking on “OK”, the device dialog shows up.

Device Dialog

Step 3:

After editing the properties of the device, you can select “Options” item under the dialog, a menu is displayed.

Device Dialog Menu

Step 4:

Choosing “Close” item and clicking on “OK”, you can actually add the device into the device list. The new device list is displayed.

If you don’t want to add the device, select “Back” item (click on the button under “Back” item) and the device list shows up (no device is added into the device list).

(2) Delete a device (DVR)

Step 1:

Selecting the device you want to delete and clicking on “OK”, a device setup menu appears:

Device Setup Menu

Step 2:

Selecting “delete device” item and clicking on “OK”, a prompt is displayed:

Prompt

Step 3:

Selecting “Yes” item under the prompt (clicking on the button under “Yes” item), the device is deleted. The device list shows up.

If you don’t want to delete the device, select “Options” item under the dialog, a menu is displayed.

Choosing “Close” item and clicking on “OK”, the deleting operation is canceled.

(3) Editing a device (DVR)

Step 1:

Selecting the device you want to edit and clicking on “OK”, a device setup menu appears:

Device Setup Menu

Step 2:

Selecting the “edit device” item and clicking on “OK”, the device dialog shows up.

Device dialog

Step 3:

After editing the properties of the device, you can select “Options” item under the dialog, a menu is displayed.

Menu

Step 4:

Choosing “Close” item and clicking on “OK”, you can actually save the device you edited into the device list. The device list is displayed.

If you don’t want to save the change, select “Back” item (click on the button under “Back” item) and the device list shows up (the device is not changed).

(4) Live view the video

Step 1:

Selecting the device you want to login and clicking on “OK”, a device setup menu appears:

Device Setup Menu

Step 2:

Selecting the “Play Video” item and clicking on “OK”, the channel list shows up.

Channel List

Step 3:

Selecting the channel you want to view and clicking on “OK”, wait for several seconds, you can live view the video if the mobile device connects the DVR successfully. Otherwise if the connection fails or you want to stop viewing, you have to choose “Back” item (click on the button under “Back” item) to quit.

Live View

(4) Live record the video

Step 1:

Selecting the device you want to login and clicking on “OK”, a device setup menu appears:

Device Setup Menu

Step 2:

Selecting the “Save Video” item and clicking on “OK”, the channel list shows up.

Channel List

Step 3:

Selecting the channel you want to record and clicking on “OK”, wait for several seconds, you can live record the video if the mobile device connects the DVR successfully. Otherwise if the connection fails or you want to stop recording, you have to choose “Back” item (click on the button under “Back” item) to quit.

(5) Remote control the camera

Step 1, 2 and 3 are the same as “Live view the video”.

Step 4:

Selecting the “Options” menu (clicking on the button under “Options” item), a menu shows up:

Step 5:

Selecting any item except “Control Off”, you can control the camera by pressing any of the left, right, up and down arrow key. A message is displayed in the window to prompt you what kind of operation you just did.

After selecting “Control Off”, you can’t control the camera anymore.