Release Notes

| Program: M2480 | Revision Number: 6.00.0324’14 Replaced Revision: 6.00.1206’13 Creation Date: 03/24/2014 |  |
| --- | --- | --- |
| Author or Designer: Peishan |  |  |
| What Changed? | WIA1.0: 6.1.1.0 Support New AVAPI protocol. WIA2.0: 2.0.1.0 Support New AVAPI protocol. |  |
| What has been Fixed? | Avscan_n.dll - Bug fixed for inside out scanning. - Support 48-bit color output. - Disable jpeg scannng in flatbed mode. - Parameter adjustment for auto orientation in fixed size. - Jpeg sampling rate 422 as default. - Jpeg quality enhancement. - Modification for auto orientation by smart image. - Bug fixes in jpeg scan with multistream. - Support blank page removal by hardware processing. - Support blank page removal by hardware processing. - Swap eop lines in duplex mode for InsideOutScan. - Jpeg scanning as default setting for D2 ASIC. - Splitting long page image in fixed size mode. - Jpeg scan mode in new AVAPI. - Support New AVAPI protocol. - Minor changes for pixel boundary. - Bug fixes for canceling scan when multifeed occured. M2480.ds - Add ColorDepth parameter. - To ignore printer status. - Reset xoffset while CAP_FEEDERENABLED is on. - Support message line feed. - Changes for capability CAP_AUTOFEED. - Changes for new AVAPI jpeg scan. - Support New AVAPI protocol. - Changes the image width with pixel bound on Citrix XenDesktop. - Be able to switch scan resolution by quality or speed mode. Twdsm_n.dll - Modification for wiif scan. - Support wifi box. - Minor changes for new AVAPI. - Minor changes for new AVAPI. - Support New AVAPI protocol. Advance.dll - Add 48bit color item in image tab. - Modify that Memory Size value don't change in setting tab. - Add ScannerCapability2(Multifeed related) with NewAvapi. - While MaxPowerOffTime = 0, hide and close power off detection. - Support Additional Length Detection with NewAvapi. - Delete Default button in information tab. - While scanner don't support BatchScan, set PageScan=1(None). - Modify source code with tab paer and tab multifeed to prevent AP crash. - Add NewScannerCapability with NewAvapi. - Modify the MoreButton String in Endorser Tab. - If there is ADF, then TWAIN UI would support multifeed with New AVAPI protocol. - Modify the String at "Resolution Define Dialog" with "75 - 600" for Brother. - Disable the Threshold item in Color Mode. - Modify the height of sample static in Endorser Tab. - Fix the wrong state of ScrollBar in Setting Tab, when clicking Default." - modify New AVAPI (DeviceAbilityVer200MinimumRes) - Support New AVAPI protocol. - Reposition the buttons of the Property Dialog. - Modify keep the same unit between paper and preview tab page." - Modify that "Unit" item in Paper tab can keep last status, when page size has be changed. |  |
| Known Defects |  |  |
| Required Files And Revision Level | File Name | Revision |
|  | Avscan_n.dll | 3.0.0.10 |
|  | M2480.ds | 3.0.0.5 |
|  | Twdsm_n.dll | 3.0.0.4 |
|  | Advance.dll | 3.0.0.13 |
|  | M2480.dll | 0.0.2013.1205 |
|  | SmartImage.dll | 1.1.27.0 |
|  | AVIP.dll | 0.0.0.50 |
|  | AVJpegEncoder.dll | 1.1.0.4 |
|  | AVScanDoubleSizeLib.dll | 1.3.0.1 |
|  | Cmatch.dll | 4.3.9.4 |
|  | AVBarcode.dll | 1.0.0.7 |
|  | Av2PDF.dll | 1.0.0.6 |
|  | AVBlank.dll | 0.1.0.3 |
|  | fpcreator_img.dll | 1.0.0.617 |
|  | AvCropEx.dll | 0.5.10.0 |
|  | Gray_quality.dll | 1.0.0.5 |
|  | LDyIp.dll | 2.0.1.11 |
|  | AVglowing.dll | 1.0.49.0 |
|  | ColorDropOut.dll | 2005/08/26 |
| Details: |  |  |